CHECK LIST FOR LAC MEETINGS

Plot Name and location 			:	 Sewree Timber Pond

Proposed for Govt. Organization	:	E-tender — E-auction

or E-tender — E-auction		:

Intended Use 				:

| Sr. No. | Check Points | Y/N/corresponding answer | Remarks |
| --- | --- | --- | --- |
| 1. | Total area of Plot & Bill Code | 136756 Sqm (As per Litho) | Part of the area is given to MMB for Yard |
| 2. | RR zone No. & CTS No. |  |  |
| 3. | Co-ordinates of plot (As per Litho) | # E 274242.114 # N 2101743.200 # E 274300.767 # N 2101807.214 # E 274699.493 # N 2101989.303 # E 274800.574 # N 2101728.980 # E 274320.529 # N 2101541.419 | Subject to demarcation on site |
| 4. | Zone as per 1991 DP | I3 |  |
| 5. | As per the Planning Proposal 2020 | Partially Commercial and partially Port allied activity zone |  |
| 6. | AS per master plan under MPA act | Commercial |  |
| 7. | FSI Permitted Base FSI Potential FSI | 1.00 as per the 1991 Development Plan and DCR, for industrial zone | As per Table No.14 of MCGM DCR 1991 |
| 8. | Custodian of Plot |  |  |
| 9. | Is PR card available? If Yes, details? |  |  |
| 10. | Reservation as per 1991 DP | NIL | Plan Attached |
| 11. | Reservation as per the Planning Proposal 2020 | Partially Garden ; Roads | Plan Attached |
| 12. | Reservation as per master plan under MPA act | Partially Exhibition Ground ; Roads | Plan Attached |
| 13. | Any hindrances / infringement /encroachment / restrictions on plot ? | Partially occupied by MMRDA |  |
| 14. | Status of Water, Sewerage, Storm water drain and Electricity Connection? |  |  |
| 15. | Road Accessibility | from existing 12 m wide approx. and proposed 30 m wide road |  |
| 16. | Specific Use sanctioned by LAC if any |  |  |
| 17. | Uses/Activity restricted on plot | As per Regulation 57 of MCGM DCR 1991 and CRZ notification 2019. | In case of Garden Reservation, 10% of the Land can be used for Sports and recreational activities with BUA of 15% of the Garden Area , Clause 23 (g), DCR 1991 |
| 18. | Allowable Setbacks | As per Regulation 29 of MCGM DCR 1991. |  |
| 19. | Height Restrictions? | As per Regulation 31 of MCGM DCR 1991. | Max. of 150 mts as per AAI subject to approval from AAI. |
| 20. | Existing Physical Condition of plot like Existing Structure, topography, any hutment around plot etc. Any road / Electricity line passing through plot? |  |  |
| 21. | Valid Structural Audit Report of structure to be allotted is available or not |  |  |
| 22. | Waterlogging issue |  |  |
| 23. | Development Authority (MCGM/SPA/KDMC) | MbPA as Special Planning Authority |  |
| 24. | List of Statutory clearance like CRZ, Heritage, Environment, MCGM/SPA, Civil Aviation Authority | CRZ clearance, MoEFCC, BMC for roads and utilities, Fire NOC from MbPA, Building permissions from MbPA as SPA, NOC from CAA for height more than 30 m. |  |
| 25. | Distance from oil installations & oil pipelines |  |  |
| 26. | Distance from Defence /Security installations | At present, there are no clear guidelines from Defense Department about restriction zone from Defense establishments. |  |
| 27. | Utilities passing below the plots like water, oil, gas pipelines, electric, optic fibre cables, etc. | Electricity Line – Water Line – Telephone Cable – Gas Pipeline – Network Connectivity | To be obtained from CED & MEED. |
| 28. | Provisions for CZMP - Zone - NDZ if any | - Falls under CRZ II - Nil |  |
| 29. | Any other relevant data |  |  |